BeeTracker Pi Stations

Student Field Operations Manual

Pi Connect Remote Access Guide

University of Portland · Dr. Tara Prestholdt · Summer 2026

This manual covers daily station management via Pi Connect. All commands must be typed exactly as shown, including dashes, underscores, and spaces. Contact bedell@up.edu for issues you cannot resolve.

Use these credentials anytime pi requests them.

Username: pi

Password: beetracker

# 1. Connecting to a Station via Pi Connect

Pi Connect lets you access any bee tracker station from any browser without needing to be on the same WiFi network.

- Open your browser and go to: connect.raspberrypi.com

- Sign in with your Raspberry Pi account credentials

- You will see a list of all 8 stations (Station-A through Station-H)

- Click the station you need and select Connect

- A terminal window opens - you are now controlling that Pi

You must be signed into the same Raspberry Pi account that was used to register the stations. Ask Charles (bedell@up.edu) if you do not have login credentials.

# 2. Daily Morning Station Check

Each morning before heading to the field, connect to each station and confirm it is running correctly.

## 2.1 Confirm detection script is running

Run this command:

ps aux | grep detect.py

You should see a line containing:

python3 /home/pi/beetracking/scripts/detect.py --site X

If you do NOT see this line, the script is not running. Go to Section 5 to restart it.

## 2.2 Check the startup log

cat ~/beetracking/data/log.txt

Look for this line near the bottom - it confirms the camera and detector started successfully:

[RUNNING] Site X - AprilTag + Color detection active.

## 2.3 Check how many detections have been recorded

cat ~/beetracking/data/site_A_detections.csv

Replace A with the correct site letter for that station. You will see one row per detection event. The header row only means no bees have been detected yet - that is normal early in the day.

# 3. Forcing a Manual Data Sync to Google Sheets

Detection data syncs automatically at the top of every hour between 10am and 6pm. If you need to push data to the Google Sheet immediately (e.g. after a busy detection period), run this command manually.

## 3.1 Navigate to the beetracking folder

cd ~/beetracking

## 3.2 Run the sync script

python3 scripts/sync_to_sheets.py --site A

Replace A with the correct site letter for that station.

You will see output like this when it works:

[SYNC] Starting sync for Site A

[SYNC] Local detections: 47

[SYNC] New rows to upload: 12

[SYNC] ✓ Sync complete - 12 new detections added to Site A tab

If you see an error about credentials or Google Sheets, contact Charles - do not attempt to fix it yourself as it involves security keys.

# 4. Viewing and Adjusting the Automatic Sync Schedule

The automatic sync schedule is controlled by crontab. You can view it to confirm the schedule, or adjust the sync hours if needed.

## 4.1 View the current schedule

crontab -l

You will see lines like these:

@reboot sleep 20 && python3 /home/pi/beetracking/scripts/detect.py --site A >> /home/pi/beetracking/data/log.txt 2>&1

0 10-18 * * * python3 /home/pi/beetracking/scripts/sync_to_sheets.py --site A >> /home/pi/beetracking/data/sync_log.txt 2>&1

## 4.2 Change the sync hours

If you need to change when syncs happen (e.g. extend to 7pm), edit the crontab:

crontab -e

This opens the nano text editor. Use arrow keys to navigate to the sync line.

The time format is:  0 START-END * * *   where START and END are hours in 24-hour format.

- 0 10-18 * * *  =  every hour from 10am to 6pm

- 0 9-19 * * *   =  every hour from 9am to 7pm

- 0 8-20 * * *   =  every hour from 8am to 8pm

After making changes press Ctrl+X, then Y, then Enter to save.

Always keep the --site X flag matching the station's assigned letter. Never change the site letter or file paths.

# 5. Restarting the Detection Script

If the detection script has stopped running (confirmed via Section 2.1), restart it manually.

## 5.1 Stop any existing instance

sudo pkill -f detect.py

This safely stops any existing detection process. If nothing was running it shows no output - that is normal.

## 5.2 Navigate to the beetracking folder

cd ~/beetracking

## 5.3 Start the script manually to test it

python3 scripts/detect.py --site A

Replace A with the correct site letter. You should see:

[CAMERA] Started at 3280x2464

[DETECTOR] AprilTag 36h11 detector ready

[RUNNING] Site A detection active. Press Ctrl+C to stop.

Press Ctrl+C to stop the manual test, then reboot the Pi so it restarts automatically via crontab:

sudo reboot

After rebooting, wait 30 seconds then reconnect via Pi Connect and run the Section 2.1 check to confirm it restarted correctly.

# 6. Rebooting a Station

A reboot is sometimes needed after script restarts, software updates, or if a station is behaving unexpectedly.

sudo reboot

Pi Connect will disconnect immediately - this is expected. Wait 45 seconds then reconnect to the station. The detection script starts automatically 20 seconds after boot.

Never pull the power cable to reboot - always use sudo reboot to prevent SD card corruption.

# 7. Checking Available SD Card Storage

Run this to see how much space is left on the SD card:

df -h /

Look at the Use% column for the / row. If it is above 80% alert Charles - the verification images may need to be cleared.

To see how much space verification images are using:

du -sh ~/beetracking/data/verification_images/

# 8. Updating Station Software from GitHub

If Charles has pushed a software update to GitHub, pull it to the station with:

cd ~/beetracking

git pull

Then restart the detection script:

sudo pkill -f detect.py

sudo reboot

Only run git pull when instructed by Charles. Never push or commit from a station Pi.

For any issue not covered here, take a screenshot of the error and contact Charles Bedell at bedell@up.edu

# Quick Reference Command Card

Cut out and keep this card at the field station for quick reference.

| Task | Command | Notes |
| --- | --- | --- |
| Check script running | ps aux ¦ grep detect.py | Should show detect.py in output |
| View startup log | cat ~/beetracking/data/log.txt | Look for [RUNNING] line |
| View detections | cat ~/beetracking/data/site_A_detections.csv | Change A to site letter |
| Force sync to Sheets | cd ~/beetracking && python3 scripts/sync_to_sheets.py --site A | Change A to site letter |
| Check sync log | cat ~/beetracking/data/sync_log.txt | Shows last sync result |
| View crontab schedule | crontab -l | Shows auto-start and sync times |
| Edit crontab schedule | crontab -e | Use Ctrl+X Y Enter to save |
| Stop detection script | sudo pkill -f detect.py | Safe stop |
| Start detection manually | cd ~/beetracking && python3 scripts/detect.py --site A | Change A to site letter |
| Reboot station | sudo reboot | Wait 45s then reconnect |
| Check SD card space | df -h / | Alert Charles if >80% |
| Check image folder size | du -sh ~/beetracking/data/verification_images/ |  |
| Update software | cd ~/beetracking && git pull | Only when instructed |
| Get station IP address | hostname -I | First number is IP |

# Troubleshooting

Problem: Pi Connect shows station as offline

Solution: The Pi may have lost power or WiFi. Check the physical station. If the power bank LED is off, the battery needs charging. Power cycle with the power bank button.

Problem: Detection script not running (no detect.py in ps aux)

Solution: Run: sudo pkill -f detect.py then sudo reboot. After 45 seconds reconnect and verify with ps aux | grep detect.py

Problem: No detections in CSV after several hours

Solution: Camera may be pointing wrong direction or too far from landing area. Physically check camera position - should be 10-15cm above the flower cluster.

Problem: Sync to Google Sheets fails

Solution: Check internet connection: ping 8.8.8.8. If ping works, contact Charles - it may be a credentials issue.

Problem: log.txt shows camera timeout error

Solution: The ribbon cable connecting the camera may be loose. Power off the Pi, reseat both ends of the ribbon cable firmly, power back on.

Problem: Station keeps rebooting or behaving erratically

Solution: Power bank may be low. Check the LED indicator on the Anker bank. Swap for a fully charged bank.

Problem: Git pull fails with authentication error

Solution: Contact Charles. Do not attempt to fix GitHub authentication yourself.